Silmulator

SIL/HIL/CIL Virtual Hardware Simulator

User Guide — Version 1.0.0
2026-05-09

# Table of Contents

1.  Introduction

2.  System Requirements

3.  Installation

4.  Quick Start

5.  Application Layout

6.  Virtual Hardware Reference

6.1  Oscilloscope — Tektronix MSO54

6.2  Power Supply — Keysight E36312A

6.3  Function Generator — Keysight 33522B

6.4  Multimeter — Keysight 34401A

6.5  MSP430 Microcontroller

6.6  Code Input Block

7.  Building a Simulation

8.  SCPI Console

9.  Saving and Loading Layouts

10.  Keyboard Shortcuts

11.  Troubleshooting

# 1. Introduction

Silmulator is a Software-in-the-Loop (SIL) / Hardware-in-the-Loop (HIL) / Code-in-the-Loop (CIL) virtual laboratory bench. Instead of wiring up physical instruments, you place virtual blocks on a canvas, connect them with wires, and interact with them using the same SCPI (Standard Commands for Programmable Instruments) that your real measurement code already sends.

This lets you test and debug firmware and measurement scripts at your desk, without physical hardware, catching command errors and response-handling bugs before they ever reach a real instrument.

## What it replaces

| Physical Equipment | Silmulator Virtual Equivalent |
| --- | --- |
| Tektronix MSO54 oscilloscope | Oscilloscope block |
| Keysight E36312A triple-output PSU | Power Supply block |
| Keysight 33522B dual-channel AWG | Function Generator block |
| Keysight 34401A 6½-digit DMM | Multimeter block |
| TI MSP430FR5994 microcontroller | MSP430 block |
| VISA/GPIB cable | Wire connection on canvas |

# 2. System Requirements

## Native Installation

| Component | Minimum Version |
| --- | --- |
| OS | macOS 12+, Windows 10, or Ubuntu 20.04+ |
| Python | 3.9 or newer |
| RAM | 512 MB |
| Disk | 300 MB |
| Display | 1024 × 768 or larger |

## Docker Installation

| Component | Requirement |
| --- | --- |
| Docker Desktop | 4.0 or newer |
| RAM (allocated to Docker) | 1 GB |
| Browser (web UI) | Chrome, Firefox, Safari, or Edge |
| Disk | 2 GB |

# 3. Installation

## Option A: pip install (recommended)

git clone <repository-url>
cd Silmulator
python3 -m venv .venv
source .venv/bin/activate    # macOS/Linux
# .venv\Scripts\activate   # Windows
pip install -e .
silmulator

## Option B: Docker — Browser Access (NoVNC)

No display setup required. The application streams to your web browser.

docker compose up -d
# Open http://localhost:6080/vnc.html
# Password: silmulator

## Option C: Docker — Native Window

Renders in a native window on your host. Requires XQuartz on macOS or X11 on Linux.

xhost +localhost   # macOS: allow Docker to connect to XQuartz
docker compose -f docker-compose.dev.yml up

## Option D: From Source with C++ Acceleration

Builds the optional high-performance C++ SCPI parser.

brew install cmake pybind11   # macOS
# apt install cmake python3-dev pybind11-dev g++  # Ubuntu
./build_cpp.sh
pip install -e .
silmulator

# 4. Quick Start

Launch the application: silmulator (or python main.py).

In the Library panel on the left, double-click Power Supply to place it on the canvas.

Double-click Multimeter to place a multimeter.

Click the output port (small circle) on the Power Supply block and drag to the input port on the Multimeter.

Right-click the Power Supply → Send SCPI command → type VOLT 5.0 → press Enter.

Right-click the Power Supply → Send SCPI command → type OUTP ON.

Right-click the Multimeter → Send SCPI command → type MEAS:VOLT:DC?

The response +5.00000000E+00 appears in the Console at the bottom.

# 5. Application Layout

The application window is divided into three areas: (1) the Library panel on the left for placing instrument blocks, (2) the Canvas in the center where blocks are arranged and connected, and (3) the SCPI Console at the bottom for typing commands and viewing responses.

# 6. Virtual Hardware Reference

## 6.1 Oscilloscope — Tektronix MSO54

4-channel mixed-signal oscilloscope with full SCPI command coverage. Supports waveform injection, measurements, and CURVE? data transfer.

| Command | Description |
| --- | --- |
| CH1:SCALE 0.5 | 500 mV per division |
| HOR:SCALE 1E-3 | 1 ms per division |
| TRIG:A:EDGE:SOURCE CH1 | Trigger source channel 1 |
| ACQ:STATE 1 | Start acquisition |
| MEASUREMENT:MEAS1:TYPE FREQUENCY | Configure measurement 1 |
| MEASUREMENT:MEAS1:VALUE? | Query measured frequency |
| CURVE? | Retrieve waveform data |
| *RST | Reset to defaults |

## 6.2 Power Supply — Keysight E36312A

Three independently controlled DC output channels with OVP/OCP protection simulation.

| Command | Description |
| --- | --- |
| INST:NSEL 1 | Select channel 1 |
| VOLT 3.3 | Set voltage to 3.3 V |
| CURR 1.0 | Current limit 1.0 A |
| OUTP ON | Enable output |
| MEAS:VOLT:DC? | Measure output voltage |
| MEAS:CURR:DC? | Measure output current |
| APPL 5.0, 2.0 | Set voltage and current in one command |

## 6.3 Function Generator — Keysight 33522B

Dual-channel arbitrary waveform generator. Generates SIN, SQU, RAMP, PULS, NOIS, DC, USER, and ARB functions with AM/FM/PM/PWM modulation, burst, and sweep.

| Command | Description |
| --- | --- |
| SOURCE1:FUNC SIN | Sine wave on channel 1 |
| SOURCE1:FREQ 1000 | 1 kHz frequency |
| SOURCE1:VOLT 2.0 | 2 Vpp amplitude |
| OUTPUT1 ON | Enable channel 1 output |
| SOURCE1:AM:STAT ON | Enable AM modulation |
| SOURCE1:SWEEP:STAT ON | Enable frequency sweep |

## 6.4 Multimeter — Keysight 34401A

6½-digit DMM supporting DC/AC voltage, DC/AC current, resistance, continuity, diode, and temperature measurements.

| Command | Description |
| --- | --- |
| MEAS:VOLT:DC? | DC voltage measurement |
| MEAS:VOLT:AC? | AC voltage (RMS) |
| MEAS:CURR:DC? | DC current |
| MEAS:RES? | 2-wire resistance |
| CONF:VOLT:DC 10, 0.001 | Configure range and resolution |
| READ? | Trigger and return one reading |
| CALC:FUNC LIM; CALC:STAT ON | Enable limit testing |

## 6.5 MSP430 Microcontroller — TI MSP430FR5994

Full register-level simulation including GPIO, 12-channel ADC, timers, UART, SPI, I2C, clocks, and power management modes.

| Peripheral | Example Command |
| --- | --- |
| GPIO | GPIO:PORT1:DIRECTION 0xFF |
| ADC | ADC:CHANNEL0:INJECT 1.65 |
| UART | UART:A0:BAUD 115200 |
| SPI | SPI:B0:TRANSFER 0xAB |
| I2C | I2C:B1:WRITE 0x48, 0x00 |
| Clocks | CLK:DCO:FREQ 16000000 |
| Power modes | PM:MODE LPM0 |

## 6.6 Code Input Block

Analyzes C, C++, or Python source code and extracts SCPI command strings. Recognized patterns include viWrite(), viQueryf(), inst.write(), inst.query(), visa_write(), scpi_send(), and ibwrt().

# 7. Building a Simulation

Place hardware blocks on the canvas from the Library panel.

Connect blocks with wires by clicking output ports and dragging to input ports.

Send SCPI commands via the right-click menu, the SCPI Console, or the Code Input block.

Observe responses in the Console panel.

Optionally inject signal data using the Python API.

Save the layout for reuse.

# 8. SCPI Console

The console at the bottom of the window accepts SCPI commands typed directly. Commands are sent to the currently selected block on the canvas. The console shows '>' for commands you send and '<' for device responses.

# 9. Saving and Loading Layouts

Layouts are saved as .sil JSON files that preserve block types, positions, wire connections, per-block device state, and Code Input content.

| Action | Shortcut |
| --- | --- |
| Save | Ctrl+S |
| Save As | (menu only) |
| Open | Ctrl+O |
| New Canvas | Ctrl+N |

# 10. Keyboard Shortcuts

| Shortcut | Action |
| --- | --- |
| Ctrl+N | New canvas |
| Ctrl+O | Open layout |
| Ctrl+S | Save layout |
| Ctrl+A | Select all blocks |
| Delete | Delete selected blocks or wires |
| Ctrl+0 | Reset zoom to 100% |
| Ctrl+Shift+F | Fit all blocks in view |
| Middle-mouse drag | Pan canvas |
| Scroll wheel | Zoom in / out |

# 11. Troubleshooting

Application does not start

Verify Python ≥ 3.9 is installed. Check that PyQt6 is present: python3 -c "import PyQt6". On Linux, install: apt install libxcb-cursor0.

SCPI Error -113: Undefined header

The command does not match any handler. Check spelling and that the command belongs to the selected device type.

Docker: browser shows blank screen

Wait 10–15 seconds for the VNC server to start. Refresh the browser.

Docker: VNC password prompt loops

Default password is 'silmulator'. To reset: docker compose down -v && docker compose up -d

Waveform CURVE? returns zeros

Run ACQ:STATE 1 first, then call inject_waveform() via the Python API to supply waveform data to the oscilloscope.